KLE Society's

KLE Technological University

Computer Networks-2

Hotel Management Case Study

Under the guidance of

Dr. Vijet Kotagi

Submitted By

Raghavendra M K       01fe20bcs133

Akash V Dalawayi       01fe20bcs154

Om Vishnudas Prabhu  01fe20bcs157

Gurutej R Abbigeri       01fe20bcs159

School of Computer Science and Engineering,

Vidyangar, Hubballi – 580031,

Academic year 2022-23

List of Chapters

| Chapter No | Chapter name | Page No |
| --- | --- | --- |
| 1 | Topology | 3-4 |
| 2 | Ip Scheme | 5-7 |
| 3 | Protocols | 8-17 |
| 4 | Financial Analysis | 18 |

1.Topology

The Case Study Hotel Management.

The hotel consists of ,

- 3 buildings.

- Each building has 3 floors.

- Each floor has 20 rooms and 5 wifi access points.

- Each room has 1 Ethernet port.

1.1 Diagram representing the topology.

1.2 Image representing implementation of     topology in Cisco Packet Tracer.

2.Ip Scheme

Central router :

CIDR: 19

Possible network: 16

| Network | Number of host | Net mask |
| --- | --- | --- |
| 10.10.0.0 | 8191 | 255.255.224.0 |
| 10.10.32.0 | 8191 | 255.255.224.0 |
| 10.10.64.0 | 8191 | 255.255.224.0 |
| 10.10.96.0 | 8191 | 255.255.224.0 |
| 10.10.128.0 | 8191 | 255.255.224.0 |

Central router 2 :

CIDR: 19

Possible network: 16

| Network | Number of host | Net mask |
| --- | --- | --- |
| 10.11.32.0 | 8191 | 255.255.224.0 |
| 10.11.64.0 | 8191 | 255.255.224.0 |
| 10.11.96.0 | 8191 | 255.255.224.0 |
| 10.11.128.0 | 8191 | 255.255.224.0 |

First building:

CIDR: 21

Possible network: 4

| Network | Number of host | Net mask |
| --- | --- | --- |
| 10.10.40.0 | 2047 | 255.255.248.0 |
| 10.10.48.0 | 2047 | 255.255.248.0 |
| 10.10.56.0 | 2047 | 255.255.248.0 |

Second building:

CIDR: 21

Possible network: 4

| Network | Number of host | Net mask |
| --- | --- | --- |
| 10.10.72.0 | 2047 | 255.255.248.0 |
| 10.10.80.0 | 2047 | 255.255.248.0 |
| 10.10.88.0 | 2047 | 255.255.248.0 |

Third building:

CIDR: 21

Possible network: 4

| Network | Number of host | Net mask |
| --- | --- | --- |
| 10.10.104.0 | 2047 | 255.255.248.0 |
| 10.10.112.0 | 2047 | 255.255.248.0 |
| 10.10.120.0 | 2047 | 255.255.248.0 |

Fourth building:

CIDR: 21

Possible network: 4

| Network | Number of host | Net mask |
| --- | --- | --- |
| 10.10.136.0 | 2047 | 255.255.248.0 |
| 10.10.144.0 | 2047 | 255.255.248.0 |
| 10.10.152.0 | 2047 | 255.255.248.0 |

3.Protocols

The protocols implemented in the topology are,

1)HTTP

The Hypertext Transfer Protocol (HTTP) is the foundation of data communication on the World Wide Web (WWW) and is used to transmit and receive data over the internet.

2)FTP

The File Transfer Protocol (FTP) is a standard network protocol used to transfer files between computers on the internet or a network

3)DHCP

Dynamic host configuration protocol used to assign ip dynamically to physical devices.

en  //This enters privileged EXEC mode on the router.

conf t //This enters global configuration mode on the router.

ip dhcp pool firstfloor //This creates a new DHCP pool named "firstfloor" on the router.

network 10.10.40.0 255.255.248.0 //This specifies the network address and subnet mask of the DHCP pool.

default-router 10.10.40.1 //This sets the default gateway for clients using this DHCP pool to the IP address 10.10.40.1.

ip dhcp excluded-address 10.10.40.1 //This specifies that the IP address 10.10.40.1 should not be assigned by the DHCP server. This address is reserved for the default gateway that was set in the previous command.

ip name-server 10.10.0.2  //This sets the DNS server for clients using this DHCP pool to the IP address 10.10.0.2.

do write memory //Save the current configuration.

ip dhcp pool secondfloor

network 10.10.48.0 255.255.248.0

default-router 10.10.48.1

ip dhcp excluded-address 10.10.48.1

ip name-server 10.10.0.2

do write memory

ip dhcp pool thirdfloor

network 10.10.56.0 255.255.248.0

default-router 10.10.56.1

ip dhcp excluded-address 10.10.56.1

ip name-server 10.10.0.2

do write memory

en

conf t

ip dhcp pool firstfloor

network 10.10.136.0 255.255.248.0

default-router 10.10.136.1

ip dhcp excluded-address 10.10.136.1

ip name-server 10.10.0.2

do write memory

ip dhcp pool secondfloor

network 10.10.144.0 255.255.248.0

default-router 10.10.144.1

ip dhcp excluded-address 10.10.144.1

ip name-server 10.10.0.2

do write memory

ip dhcp pool thirdfloor

network 10.10.152.0 255.255.248.0

default-router 10.10.152.1

ip dhcp excluded-address 10.10.152.1

ip name-server 10.10.0.2

do write memory

4)DNS

The Domain Name System (DNS) is a hierarchical decentralized naming system for computers, services, or any resource connected to the Internet or a private network.

5)SMTP

The Simple Mail Transfer Protocol (SMTP) is a standard protocol used for sending and receiving email messages between servers and clients.

6)RIP

The Routing Information Protocol (RIP) is a distance-vector routing protocol used in local area networks (LANs) and wide area networks (WANs) to determine the best path for data to travel between two nodes or devices.

Example of code snippet to enable RIP in first building router:

router rip

version 2

network 10.10.40.0

network 10.10.32.0

network 10.10.48.0

network 10.10.56.0

network 10.11.32.0

do write memory

The router rip command enables RIP routing on the router. The version 2 command specifies that RIP version 2 should be used. This version includes support for variable-length subnet masks (VLSM) and multicast updates.

The network commands specify the network addresses that should be advertised using RIP. In this configuration, five networks are being advertised: 10.10.40.0, 10.10.32.0, 10.10.48.0, 10.10.56.0, and 10.11.32.0.

The do write memory command saves the current configuration to non-volatile memory, so that it will be retained after a reboot.

7) NAT

In Dynamic NAT, IP addresses are dynamically mapped to each other on a one-to-one basis as per the needs. It establishes a mapping between an Inside Local IP address and a pool of Global IP addresses.

Central routers were NAT enabled routers

commands:

en

conf t

Configure the router’s outside interface

int f0/6

ip nat outside

exit

Configure the router’s inside interface

int f0/0

ip nat inside

exit

int f0/1

ip nat inside

exit

int f0/9

ip nat inside

exit

int f0/8

ip nat inside

exit

Configure the pool of global IP addresses

ip nat pool pool1 72.10.11.2 72.10.11.6 netmask 255.255.255.0

Configure access-list that has list of inside address that will be translated

access-list 1 permit 10.10.0.0  0.0.255.255

Enable the NAT

ip nat inside source list 1 pool pool1

x

4.Financial Analysis

| SI.No | Equipment | per unit cost | quantity | cost |
| --- | --- | --- | --- | --- |
| 1 | Router,8 ports | 3,30,739Rs | 6 | 19,84,434/- |
| 2 | 1Gbps ethernet cable | 40Rs /m | 570m | 22,800/- |
| 3 | 100Mbps ethernet cable | 25Rs/m | 19020m | 4,75,500/- |
| 4 | Dell R730xd server | 1,87,950Rs | 1 | 1,87,950/- |
| 5 | Switch,48 ports | 1,10,000Rs | 13 | 14,30,000/- |
| 6 | Access point | 20,351Rs | 60 | 12,21,060/- |
| Total |  |  |  | 53,21,744/- |

Service charge 12% =6,38,609/-

Total cost=59,60,353/-